Change/tighten/loosen collet in Carlo Di Ge Giorgi Handpiece

- Insert a pin into the hole at the base of the handpiece, ensuring it threads all the way through. You may need to hold this in position:

- Unscrew black cone piece from top of handpiece:

- Open the lever to right degree angle, which will also open the collet:

- Insert a 1.5mm allen key into the collet and rotate a quarter turn anti-clockwise. You should feel a screw loosen.

- Once loose, the collet should be able to be loosened in and out by hand. You are now able to remove it entirely by rotating the collet anti-clockwise. You are able to tighten it by rotating it clockwise:

- Once the collet is changed or loosened/tightened to desired amount, insert the allen key back into the collet, and tighten the internal screw by turning clockwise. You will feel the screw bottom out.  Do not overtighten.

- At this point you are able to close the lever to the down position, screw the black cone back on and remove the pin from the base of the handpiece.